API Interface Manual for Control Card Development（Java）

Version: V1.2.3

2019.04.26

Revised records：

| Date | Version | Modified Contents | Author |
| --- | --- | --- | --- |
| 2019-4-26 | V1.0.0 | Initial version |  |
| 2019-5-8 | V1.1.0 | Split windows and monitor connection status |  |
| 2019-5-24 | V1.2.0 | Send program by protocol（MultiWindow） |  |
| 2019-5-24 | V1.2.1 | Add send scaled picture |  |
| 2019-6-28 | V1.2.2 | Add sendMulti interface |  |
| 2019-8-22 | V1.2.3 | 1. Add timely switch screen interface; 2. Added an interface to exit the multi-window program display. Fix some known issues. |  |

# 1、The interface of send programs

## com.lumen.ledcenter3.protocol.ExtSendUtil

com.lumen.ledcenter3.protocol.ExternalNetworkSendProtocol.OnTcpNetWorkListener

## 1.1、Overview

ExtSendUtil

|  | Name | Description | Complete |
| --- | --- | --- | --- |
| 1 | initNetwork |  | Completed |
| 2 | setListener |  | Completed |
| 3 | splitScreen |  | Completed |
| 4 | sendText |  | Completed |
| 5 | sendPicture |  | Completed |
| 6 | sendPictureWithScale |  | Completed |
| 7 | saveSentDataToFlash |  | Completed |
| 8 | clearSavedDataFromFlash |  | Completed |
| 9 | getBrightness |  | Completed |
| 10 | setBrightness |  | Completed |
| 11 | setScreenOnAndOffTime |  | Completed |
| 12 | getScreenOnAndOffTime |  | Completed |
| 13 | exitSetScreenOnAndOffTime |  | Completed |
| 14 | sendMulti |  | Completed |

OnTcpNetWorkListener

| No | Name | Description | Complete |
| --- | --- | --- | --- |
| 1 | onSocketInit |  | Completed |
| 2 | onStatus |  | Completed |
|  | onBackBytes |  | Completed |
|  | onTcpProcess |  | Completed |
|  | breakSocket |  | Completed |

## 1.2、Detail description

### initNetwork

| initNetwork (String ip, int port, String idcode) |  |
| --- | --- |
| Use |  |
| Params | Ip : control card’s network ip address Port: net port , default 5200 Idcode: default 255.255.255.255 |

### setListener

| setListener (OnTcpNetWorkListener listener) |  |
| --- | --- |
| Use |  |
| Params | Listener: Monitor the network communicate status and progress |

### splitScreen

| boolean splitScreen(int windowCount, int[]... winRects) |  |
| --- | --- |
| Use |  |
| Params | windowCount: The number of windows to be split, and within the limits of 1 to 8. winRects：array of windows’s rects. winRect is The coordinates of a window. Each window uses four integers to represent its coordinate values of "left, top, right and bottom" |
| Return | True: successfully，false otherwise. |
| example |  |
| notice | This method is time-consuming, you can call it in work thread to avoid blocking of main thread, and monitor the result int OnTcpNetWorkListener.onStatus |

### sendText

| boolean sendText( int nWndNo, String content, int crColor, int nFontSize, int nSpeed, int nEffect, int nStayTime, int nAlignmentHori, int nAlignmentVert) |  |
| --- | --- |
| Use |  |
| Params | nWndNo: Sequence number of window, virtual value 0-7. Content: text string. crColor: value 1-7 for color: red, green, yellow, blue, purple, blue and white. nFontSize: = 0 means 8 words; = 2 means 16 words; = 3 means 24 words; = 4 means 32 words; = 5 means 40 words; = 6 means 48 words; = 7 means 56 words. nSpeed: value 1 ~ 100. The smaller the value, the faster the speed. nEffect: Display Effect nStayTime: The unit is seconds. nAlignmentHori: Horizontal alignment (0: left alignment, 1: horizontal middle, 2: right alignment) nAlignmentVert: Vertical alignment (0: top alignment, 1: vertical middle, 2: bottom alignment) |
| Return | True: successfully，false otherwise. |
| example |  |
| notice | This method is time-consuming, you can call it in work thread to avoid blocking of main thread, and monitor the result int OnTcpNetWorkListener.onStatus |

### sendPicture

| boolean sendPicture(int nWndNo, int nSpeed, int nEffect, int nStayTime, String picturePath) |  |
| --- | --- |
| Use |  |
| Params | nWndNo: Sequence number of window, virtual value 0-7. nSpeed: value 1 ~ 100. The smaller the value, the faster the speed. nEffect: Display Effect nStayTime: The unit is seconds. picturePath: absolute path of picture file. Include gif, jpg, png. |
| Return | True: successfully，false otherwise. |
| example |  |
| notice | This method is time-consuming, you can call it in work thread to avoid blocking of main thread, and monitor the result int OnTcpNetWorkListener.onStatus |

### sendPictureWithScale

| boolean sendPicture(int nWndNo, int winWidth, int winHeight, int nSpeed, int nEffect, int scaleType, int nStayTime, String picturePath) |  |
| --- | --- |
| Use |  |
| Params | nWndNo: Sequence number of window, virtual value 0-7. winWidth: width of window winHeight: height of window nSpeed: value 1 ~ 100. The smaller the value, the faster the speed. nEffect: Display Effect scaleType: picture scale type nStayTime: The unit is seconds. picturePath: absolute path of picture file. Include gif, jpg, png. |
| Return | True: successfully，false otherwise. |
| example |  |
| notice | This method is time-consuming, you can call it in work thread to avoid blocking of main thread, and monitor the result int OnTcpNetWorkListener.onStatus |

### sendMulti

| boolean sendMulti (int itemNum, String[] texts, int[] colors, int[] fontSizes, int[] showEffects) |  |
| --- | --- |
| Use | Send multi texts |
| Params | itemNum : item count texts: text contents colors: text color for each content. value 1-7 for color: red, green, yellow, blue, purple, blue and white. fontSizes: text size for each content showEffects: text showEffect for each content |
| Return | True: successfully，false otherwise. |
| example |  |
| notice | This method is time-consuming, you can call it in work thread to avoid blocking of main thread, and monitor the result int OnTcpNetWorkListener.onStatus |

### saveSentDataToFlash

| boolean saveSentDataToFlash() |  |
| --- | --- |
| Use | Save split screen information and programs which sent by sendText and sendPicture. If not call this, the sent data will not be saved when control card restart. |
| Params |  |
| Return | True: successfully，false otherwise. |
| notice | This method is time-consuming, you can call it in work thread to avoid blocking of main thread, and monitor the result int OnTcpNetWorkListener.onStatus |

### clearSavedDataFromFlash

| boolean clearSavedDataFromFlash() |  |
| --- | --- |
| Use | Clear Save split screen information and programs which sent by sendText and sendPicture. And the sent data will not be saved when control card restart. |
| Params |  |
| Return | True: successfully，false otherwise. |
| notice | This method is time-consuming, you can call it in work thread to avoid blocking of main thread, and monitor the result int OnTcpNetWorkListener.onStatus |

### getBrightness

| int[] getBrightness() |  |
| --- | --- |
| Use |  |
| Params |  |
| Return | The brightness value, length is 24, represent 24 time slot from 00:00. |
| notice | This method is time-consuming, you can call it in work thread to avoid blocking of main thread, and monitor the result int OnTcpNetWorkListener.onStatus |

### setBrightness

| boolean setBrightness(int[] brigghtnessValue) |  |
| --- | --- |
| Use |  |
| Params | brigghtnessValue: The brightness value, length is 24, represent 24 time slot from 00:00. When all 24 value is -1, then is in photosensitivity mode. |
| Return | True: successfully，false otherwise. |
| notice | This method is time-consuming, you can call it in work thread to avoid blocking of main thread, and monitor the result int OnTcpNetWorkListener.onStatus |

### setScreenOnAndOffTime

| boolean setScreenOnAndOffTime(int nHourOn, int nMinuteOn, int nHourOff, int nMinuteOff) |  |
| --- | --- |
| Use |  |
| Params | nHourOn: 0~23 nMinuteOn: 0~59 nHourOff: 0~23 nMinuteOff: 0~59 |
| Return | True: successfully，false otherwise. |
| notice | This method is time-consuming, you can call it in work thread to avoid blocking of main thread, and monitor the result int OnTcpNetWorkListener.onStatus |

### getScreenOnAndOffTime

| int[] getScreenOnAndOffTime () |  |
| --- | --- |
| Use |  |
| Params |  |
| return | The screen on and off time, length is 4. When value is 255, explain that the switch screen settings are not turned on. |
| notice | This method is time-consuming, you can call it in work thread to avoid blocking of main thread, and monitor the result int OnTcpNetWorkListener.onStatus |

### exitSetScreenOnAndOffTime

| boolean exitSetScreenOnAndOffTime () |  |
| --- | --- |
| Use | Exit screen on and off settings. |
| Params |  |
| Return | True: successfully，false otherwise. |
| notice | This method is time-consuming, you can call it in work thread to avoid blocking of main thread, and monitor the result int OnTcpNetWorkListener.onStatus |

### setScreenSwitch

| boolean setScreenSwitch(boolean turnOn) |  |
| --- | --- |
| Use | Switch the screen immediately (restart card is invalid) |
| Params | TurnOn : true means ON，false means OFF |
| Return | True: successfully，false otherwise. |
| notice | This method is time-consuming, you can call it in work thread to avoid blocking of main thread, and monitor the result int OnTcpNetWorkListener.onStatus |

### quitExternalScreen

| boolean quitExternalScreen() |  |
| --- | --- |
| Use | Exit multi-window program mode and return to regular program playback |
| Params |  |
| Return | True: successfully，false otherwise. |
| notice | This method is time-consuming, you can call it in work thread to avoid blocking of main thread, and monitor the result int OnTcpNetWorkListener.onStatus |

OnTcpNetWorkListener

### onSocketInit

| onSocketInit(int result); |  |
| --- | --- |
| Use | Monitor socket connect status |
| Params | Result: 0 fail,1 success |

### onStatus

| onStatus(int status, int socketindex); |  |
| --- | --- |
| Use | Monitor data send status |
| Params | Status: -1 means error occur , 1 means ok. Socketindex: socket Index |

### onBackBytes

| onBackBytes(int[] backbytes, int socketindex); |  |
| --- | --- |
| Use |  |
| Params | Backbytes: back data from control card Socketindex：socket Index |

### onTcpProcess

| onTcpProcess(int process, int totalprocess, int socketindex); |  |
| --- | --- |
| Use | Monitor data send progress when send picture or other files |
| Params | Process： Totalprocess： Socketindex：socket Index |

### breakSocket

| breakSocket(int socketindex); |  |
| --- | --- |
| Use | Monitor socket closed. |
| Params | Socketindex: socket Index |

## 1.3、Demo

# 2、Supplementary to parameter description

## 2.1、Color and grey scale

| Name | Value | Constant name（in ProtocolConstant class） |
| --- | --- | --- |
| Mono | 0x01 | COLOR_TYPE_MONO |
| Mono with 256-grade grey scale | 0x71 | COLOR_TYPE_MONO_WITH_GRAY |
| Dual colors | 0x03 | COLOR_TYPE_RG_COLOR |
| Dual colors with 256-grade grey scale | 0x73 | COLOR_TYPE_RG_COLOR_WITH_GRAY |
| RGB color | 0x07 | COLOR_TYPE_RGB_COLOR |
| Full color | 0x77 | COLOR_TYPE_FULL_COLOR |

## 2.2、Text and Picture Display Effect

example：ShowEffect.Random.getEffect();

Random

Instant

Open_left

Open_right

Open_horizontal

Open_vertical

Shutter_vertical

Shift_left

Shift_right

Shift_up

Shift_down

Scroll_up

Scroll_left

Scrollleft_continuously

Scroll_right

Scroll_right_continuously

Blink

Shutter_horizontal

Open_clockwise

Open_anticlockwise

Windmill_clockwise

Windmill_anticlockwise

Rectangle_out

Rectangle_in

Corner_out

Corner_in

Round_out

Round_in

Open_top_left

Open_top_right

Open_bottom_left

Open_bottom_right

Open_slash

Open_backslash

Slide_top_left

Slide_top_right

Slide_bottom_left

Slide_bottom_right

Open_cross_in

Open_cross_out

Zebra_cross_horizontal

Zebra_cross_vertical

Mosaic_large

Mosaic_small

Laser_line_upward

Laser_line_downward

Scrape_up

Drop_down

Slide_left_right

Slide_top_bottom

Slewing_out

Slewing_in

Chessboard_horizontal

Chessboard_vertical

Scroll_up_continuously

Scroll_down_continuously

Expand_from_top

Expand_from_bottom

Expand_vertical

Blind_horizontal

Blind_vertical

Snow_fall

Scroll_down

Open_left_right

Open_up_down

Open_2_fan

Slide_zebra_horizontal

Slide_zebra_vertical

## 2.3、picture scale type

| Value | Type | Constant name（ProtocolConstant） |
| --- | --- | --- |
| 0 | Center | SHOWMODE_CENTER |
| 1 | Scale | SHOWMODE _SCALE |
| 2 | Stretch | SHOWMODE _STRETCH |
| 3 | Tiled | SHOWMODE _TILED |

## 2.4、font size

Note: The dot matrix size of extended fonts supported by each control card is different. If the text is not displayed in a certain size, the dot matrix size is not supported.

| Value | Size（pixel） | Constant name（ProtocolConstant） |
| --- | --- | --- |
| 0 | 8 | FONTSIZE_8 |
| 1 | 12 | FONTSIZE_12 |
| 2 | 16 | FONTSIZE_16 |
| 3 | 24 | FONTSIZE_24 |
| 4 | 32 | FONTSIZE_32 |
| 5 | 40 | FONTSIZE_40 |
| 6 | 48 | FONTSIZE_48 |
| 7 | 56 | FONTSIZE_56 |

## 2.5、socket Index

SOCKETINDEX_SPLIT_SCREEN = 2;

SOCKETINDEX_SEND_TEXT = 3;

SOCKETINDEX_SEND_PICTURE = 4;

SOCKETINDEX_SAVE_DATA = 5;

SOCKETINDEX_CLEAR_DATA = 6;

SOCKETINDEX_GET_BRIGHTNESS = 7;

SOCKETINDEX_SET_BRIGHTNESS = 8;

SOCKETINDEX_SET_SCREEN_ONOFF = 9;

SOCKETINDEX_GET_SCREEN_ONOFF = 10;

SOCKETINDEX_EXIT_SET_SCREEN_ONOFF = 11;